Private Cloud Networking Architecture

1. Introduction

A private cloud network is a data center–level infrastructure designed to provide scalable, secure, and high-performance compute and storage services within an organization. Modern private clouds commonly adopt a leaf–spine (Clos) architecture to achieve low latency, high bandwidth, and horizontal scalability.

2. Data Center Network Architecture

Leaf–Spine Topology Overview

Key Components

2.1 Top-of-Rack (ToR) Switch

- Installed within each rack

- Connects directly to servers

- Functions as an access layer switch (Layer 2 or Layer 3)

- Provides uplinks to multiple spine switches

2.2 Spine Switch

- Forms the backbone of the data center network

- Connects all ToR switches

- Operates typically at Layer 3

- Enables equal-cost multipath routing (ECMP)

2.3 Servers (Compute Nodes)

- Host virtual machines or containers

- Execute application workloads

- Participate in east–west traffic communication

3. Functional Roles

3.1 ToR Switch (Access Layer)

- Connects servers within a rack

- Handles VLAN tagging and local switching

- Aggregates traffic and forwards it to spine switches

3.2 Spine Switch (Aggregation/Core Layer)

- Interconnects all ToR switches

- Ensures high-speed routing across racks

- Provides multiple equal-cost paths for traffic

3.3 Communication Flow

Traffic between servers in different racks follows:

Server → ToR Switch → Spine Switch → ToR Switch → Server

4. Detailed IP Addressing Design

A hierarchical IP addressing scheme is used to separate physical infrastructure (underlay) and virtual workloads (overlay).

4.1 Underlay Network (Physical Infrastructure)

Used for switch-to-switch and switch-to-server communication.

Spine Layer

Spine1 → 10.0.0.1/24  
Spine2 → 10.0.0.2/24

ToR Layer (Per Rack)

| Rack | ToR IP | Subnet |
| --- | --- | --- |
| Rack 1 | 10.0.1.1 | 10.0.1.0/24 |
| Rack 2 | 10.0.2.1 | 10.0.2.0/24 |
| Rack 3 | 10.0.3.1 | 10.0.3.0/24 |

4.2 Server-Level Addressing (Node Level)

Each server connected to a ToR switch receives an IP within the rack subnet.

Example: Rack 1

| Server | IP Address |
| --- | --- |
| Node1 | 10.0.1.10 |
| Node2 | 10.0.1.11 |
| Node3 | 10.0.1.12 |

4.3 Overlay Network (Pod / VM Level)

Overlay networks are independent of physical topology and use VXLAN.

| Node | Pod CIDR |
| --- | --- |
| Node1 | 192.168.1.0/24 |
| Node2 | 192.168.2.0/24 |
| Node3 | 192.168.3.0/24 |

Example Pods

| Pod | IP |
| --- | --- |
| Pod A | 192.168.1.2 |
| Pod B | 192.168.2.3 |

4.4 Service Network

Used for internal service discovery in Kubernetes.

Service CIDR: 10.96.0.0/12

5. End-to-End Communication Flow

5.1 Same Rack Communication

Pod A → Node → ToR → Node → Pod B

- No spine switch involved

- Low latency

5.2 Cross-Rack Communication

Pod A  
→ Node1  
→ ToR1  
→ Spine  
→ ToR2  
→ Node2  
→ Pod B

Detailed Flow Explanation

- Pod A sends packet to destination Pod B

- Packet encapsulated using VXLAN

- Sent to Node1 network interface

- Forwarded to ToR1

- Routed via Spine switch

- Delivered to ToR2

- Sent to Node2

- Decapsulated and delivered to Pod B

5.3 External Communication (North–South)

Client → Load Balancer → ToR → Node → Pod

6. Traffic Types

6.1 East–West Traffic

- Internal communication within data center

- Example: Pod-to-pod

6.2 North–South Traffic

- External communication

- Example: User accessing application

7. Routing and Protocols

7.1 Equal-Cost Multi-Path (ECMP)

- Distributes traffic across multiple spine switches

- Provides redundancy and load balancing

7.2 Border Gateway Protocol (BGP)

- Used in underlay network

- Enables scalable routing between switches

8. Network Virtualization

VXLAN

- Encapsulates Layer 2 frames into Layer 3 packets

- Enables cross-rack communication

- Supports multi-tenant environments

9. Simulation Architecture Mapping

Simulation Tools

- Containerlab

- Docker

- Kubernetes

Mapping Table

| Real Infrastructure | Simulation Equivalent |
| --- | --- |
| Spine Switch | Container (router) |
| ToR Switch | Container (switch/router) |
| Server | Kubernetes node |
| Pod | Container |

10. Monitoring and Metrics

Key Metrics

- CPU utilization per node and pod

- Memory utilization

- Network throughput

- Latency

Tools

- kubectl top

- Prometheus

- Grafana

11. Security Considerations

- VLAN/VXLAN-based segmentation

- Access control lists and firewall policies

- Kubernetes Network Policies

12. Advantages of Leaf–Spine Architecture

- Predictable latency (1–2 hops)

- High bandwidth availability

- Horizontal scalability

- Fault tolerance

13. Limitations in Simulation

- Shared CPU and memory in local environments

- Lack of physical isolation

- Limited bandwidth compared to real data centers

Despite these limitations, simulation accurately represents logical behavior.

14. Conclusion

Private cloud networking uses a leaf–spine architecture combined with underlay and overlay networking to enable scalable and efficient communication. A structured IP addressing scheme across spine, rack, server, and pod levels ensures proper routing and isolation. Simulation tools such as Containerlab, Docker, and Kubernetes provide a practical platform to replicate and analyze this architecture.